CowNect Collar Firmware

System Overview Report

Firmware version: 0.1.0-proto

Platform: ESP32-S3-WROOM-1-N16R8, ESP-IDF v5.5.5, C++

Date: 24 September 2026

Contents

1. Executive Summary	3

2. System Operation	3

3. Hardware Overview	3

4. Software Architecture	4

4.1 Design rules	4

5. Start-up and Operating Modes	5

6. Sensor Capture	5

6.1 Shared ADC	5

6.2 Health tracking	5

6.3 Identity and time	5

7. Data Processing and Communication	6

7.1 Telemetry summary (Material 11)	6

7.2 Serialisation (Material 13)	6

7.3 Communication modes	6

7.4 LoRa radio	6

8. Power Management and Timing	6

9. Current Status and Open Items	7

9.1 Values not yet configured	7

9.2 Development defaults	7

9.3 Hardware questions	7

9.4 Out of scope	7

10. Bring-up Procedure	7

(If the contents list is empty, right-click it and choose Update Field.)

# 1. Executive Summary

This report explains how the CowNect collar firmware is structured and how it is designed to work. It is written for team members who need an overall understanding of the system without reading the source code.

The collar is a battery-powered sensor node worn by a cow. Every 120 seconds it records its sensors for 30 seconds, summarises the recording, sends data to the receiving side (LoRa gateway and/or Jetson over Wi-Fi), and then deep-sleeps until the next cycle.

Prototype status. The firmware compiles cleanly with ESP-IDF v5.5.5, but it has not yet run on the physical collar PCB. Everything in this report describes the intended design. Timings, thresholds and radio behaviors still require hardware validation, and several network and radio settings have deliberately not been configured yet (Section 9).

Related documents: README.md (build steps and test list), BUILD_AND_VALIDATION_REPORT.md (test results), docs/protocols.md (byte-level packet formats), hardware_questions.md (open hardware questions).

# 2. System Operation

The collar repeats a fixed 120-second cycle:

| Step | Phase | What happens |
| --- | --- | --- |
| 1 | Power on | Switch on the sensor power rail and wait for it to stabilize. |
| 2 | Capture (30 s) | Record accelerometer, GPS, microphone, board temperature and cow temperature at the same time. |
| 3 | Features | Reduce the recording to a small telemetry summary (averages, RMS, position, temperatures, health flags). |
| 4 | Communicate | Send the summary and/or the full raw recording, depending on the configured communication mode. |
| 5 | Power off | Switch off the sensor power rail. |
| 6 | Deep sleep | Sleep for whatever remains of the 120 s cycle, then start again. |

Table 1. The 120-second collar cycle

On the receiving side, a Heltec LoRa gateway (Material 16) and a Jetson receiver (Material 17) decode the data. Both are separate projects and are not part of this firmware.

# 3. Hardware Overview

| Block | Part | Interface | ESP32-S3 pins |
| --- | --- | --- | --- |
| MCU | ESP32-S3-WROOM-1-N16R8 (16 MB flash, 8 MB PSRAM) | - | - |
| Accelerometer | LIS2DW12 (50 Hz, ±4 g) | I2C, addr 0x18 | SDA 6, SCL 7 |
| GPS | ATGM336H (9600 baud, 1 Hz) | UART | RX 17, TX 18 |
| Microphone | Analog microphone + MCP6001 amplifier | ADC1 | GPIO 5 |
| Board temperature | MCP9700 | ADC1 | GPIO 1 |
| Cow temperature | MF58 10 kΩ NTC thermistor (divider) | ADC1 | GPIO 2 |
| LoRa radio | Ra-01SH (SX1262), external antenna | SPI | CS 9, MOSI 10, SCK 11, MISO 12, RST 13, DIO1 14, BUSY 21 |
| Sensor power rail | TPS22919 load switch (SWITCHED_3V3) | GPIO | PERIPH_EN 4 |
| Mode switch | PROG switch | GPIO | 15 (LOW = programming, HIGH = normal) |

Table 2. Hardware blocks and pin assignment

All pin numbers are defined in one file, components/board/include/board_pins.h. No other file hard-codes a GPIO number.

# 4. Software Architecture

The firmware is written in C++ on ESP-IDF and is divided into components. Each layer only calls the layers below it.

| Layer | Components | Responsibility |
| --- | --- | --- |
| Entry | main/app_main.cpp, cownect_app | Bring-up checklist and the final firmware entry point run_cownect_firmware(). |
| Control | scheduler, power | Runs the 120 s cycle; switches the sensor rail; enters deep sleep. |
| Work | sensor_manager, features, communication | Runs the 30 s capture; computes the telemetry summary; sends data (comm_manager, lora_full, hybrid, wifi_link, tcp_client, wifi_upload). |
| Data | data, capture_stream | Holds the capture in memory (CaptureSession), keeps counters across deep sleep, and serialises the capture with CRC32. |
| Drivers | accelerometer, gps, temperature, microphone, analog_adc, lora_radio | Talk to individual chips. analog_adc is the shared continuous ADC engine; lora_radio wraps the SX1262. |
| Board | board | Pins, mode switch, PERIPH_EN, I2C bus, ADC1 ownership and calibration, PSRAM check. |
| Configuration | cn_config | menuconfig options and central constants, used by all layers. |
| Test | devtest | Bring-up tests and development console; uses the real production code. |

Table 3. Software layers (top to bottom)

## 4.1 Design rules

One owner per resource: the board layer owns ADC1, the I2C bus and the power-enable pin. Drivers request access from it and never configure these directly.

One set of constants: timing, sample rates and buffer sizes live in cn_config. Tunable values appear in idf.py menuconfig under "CowNect collar configuration".

Tests use production code: bring-up tests and console commands call the same drivers and managers as the real firmware. There are no separate test drivers.

No guessed values: any value that must come from the hardware or the network is left as CONFIG_NOT_SET. The feature that needs it refuses to run and reports BLOCKED instead of running on a guess.

# 5. Start-up and Operating Modes

main/app_main.cpp is a bring-up checklist rather than the final program. By default it only performs a safe base initialization with the sensor rail commanded OFF, prints a summary and idles. During PCB bring-up, tests are enabled one at a time (Materials 4 to 15). The last line, run_cownect_firmware(), starts the real firmware.

When the real firmware starts, it reads the PROG switch (GPIO 15, debounced):

| Mode | Behaviour |
| --- | --- |
| PROGRAMMING (switch LOW) | Development console on the serial port. Nothing starts automatically; the operator types "test ..." commands. |
| NORMAL (switch HIGH) | The 120 s production cycle starts automatically and repeats through deep sleep. |
| UNSTABLE (not settled) | Stays awake and re-checks. Never enters deep sleep, so a floating switch cannot lock the device out. |

Table 4. Operating modes

# 6. Sensor Capture

The SensorManager runs one 30-second capture and stores all results in a single CaptureSession.

| Sensor | Rate | Samples in 30 s | How it is read |
| --- | --- | --- | --- |
| Accelerometer | 50 Hz | ~1,500 | LIS2DW12 FIFO, polled every 100 ms over I2C |
| GPS | 1 Hz | up to 40 fixes | UART NMEA stream, parsed in firmware |
| Microphone | 32 kHz, 12-bit | ~960,000 (~1.9 MB) | Continuous ADC with DMA, stored in PSRAM |
| Board temperature | 1 Hz (reported) | ~30 | Shared continuous ADC, averaged into 1 s values |
| Cow temperature | 1 Hz (reported) | ~30 | Shared continuous ADC, averaged into 1 s values |

Table 5. Sensor capture rates

## 6.1 Shared ADC

The microphone and both temperature inputs are all on ADC1. One continuous ADC engine therefore samples at 64,000 conversions per second in the repeating pattern MIC, BOARD_TEMP, MIC, COW_TEMP. The microphone receives 32 kHz; the temperature channels are averaged into one reading per second and then converted with the normal temperature code (MCP9700 linear formula, NTC Beta formula). Readings use the ESP-IDF calibration driver. The 12 dB attenuation setting is a provisional bring-up value.

## 6.2 Health tracking

Each sensor records its own status: OK, degraded (for example FIFO or DMA overflow, or no GPS fix) or failed. A capture can complete with some sensors degraded; the status flags travel with the data so the receiver knows how far to trust each part.

## 6.3 Identity and time

Each capture carries a device_id (currently the development default 0x00000001) and an incrementing capture_id. The capture_id is stored in RTC memory, so it survives deep sleep but not a full power loss. Timestamps are monotonic microseconds since boot; no real clock time is used because no absolute time source has been configured.

# 7. Data Processing and Communication

## 7.1 Telemetry summary (Material 11)

SimpleFeatureProcessor reduces the capture to a TelemetryRecord containing: the last valid GPS position, mean board and cow temperature, accelerometer magnitude mean/RMS/min/max, microphone mean and AC RMS, and status flags such as GPS_NO_VALID_FIX, COW_PROBE_FAULT and CYCLE_OVERRUN.

## 7.2 Serialisation (Material 13)

To send the full raw capture, the capture_stream component encodes it as "capture stream v1": a 68-byte header followed by six typed sections (accelerometer, GPS, board temperature, cow temperature, microphone, diagnostics). All fields are explicit little-endian; no raw C structure is ever transmitted. The whole stream is protected by CRC-32 (zlib variant). Data is encoded directly from the capture buffers while it is being sent, so no second full-size copy is held in memory.

## 7.3 Communication modes

| Mode | What is sent | Path |
| --- | --- | --- |
| Disabled (current default) | Nothing | - |
| LORA_FULL | The entire capture stream | LoRa, split into fragments; stop-and-wait with acknowledgement and retry per fragment |
| HYBRID_LORA_WIFI | 1. 49-byte telemetry packet 2. Entire capture stream | 1. LoRa to the Heltec gateway 2. Wi-Fi TCP to the Jetson |

Table 6. Communication modes (selected in menuconfig)

The hybrid mode is the intended main path: the small summary goes over long-range LoRa, and the large raw recording (about 2 MB, mostly microphone data) goes over Wi-Fi. LORA_FULL exists to test whether full transfers over LoRa alone are practical; because of LoRa data rates it is expected to take much longer than the 120 s cycle.

## 7.4 LoRa radio

The SX1262 radio uses a pinned copy of Semtech's official sx126x_driver, wrapped behind the project's own HAL and adapter (ILoraRadio). The rest of the firmware never talks to the chip directly. Every radio operation has a timeout, so a stuck radio cannot hang the cycle.

RF safety. The firmware refuses to transmit until the complete RF profile has been configured and the operator has set COWNECT_LORA_ANTENNA_VERIFIED to confirm that the external antenna is attached. Transmitting without an antenna can damage the radio.

# 8. Power Management and Timing

PowerManager switches the peripheral rail (SWITCHED_3V3) and, after switching it on, waits a fixed stabilisation delay (development default 100 ms).

The rail stays on during communication while LoRa needs it. In hybrid mode it can be switched off before the Wi-Fi upload, because Wi-Fi is inside the ESP32-S3.

SleepManager uses timer-only deep sleep. It measures how long the device has been awake and sleeps for the remainder of the 120 s. If a cycle ran over 120 s, it records an overrun and starts the next cycle immediately.

Per-cycle metrics (capture time, communication time, awake time, overrun) are logged and kept in RTC memory.

# 9. Current Status and Open Items

## 9.1 Values not yet configured

These values have not been supplied yet. Every feature that depends on one reports CONFIG_NOT_SET or BLOCKED:

LoRa RF profile (frequency, bandwidth, spreading factor, coding rate, power, sync word, TCXO/DIO2, regulator, timeouts) and the antenna-confirmed flag

LORA_FULL fragment size, ACK timeout and retry count

Wi-Fi SSID and password, Jetson IP address and port, network timeouts

Cow thermistor open/short detection thresholds

Absolute time source

## 9.2 Development defaults

The following are temporary values, labelled NEEDS_HARDWARE_VALIDATION: device_id 0x00000001, 100 ms rail stabilisation, 50 ms mode-switch debounce, ADC attenuation 12 dB, 12-bit resolution.

## 9.3 Hardware questions

Questions about the power switch and ECO jumper behaviour, the TCXO, the DIO2 RF switch and other hardware details are tracked in hardware_questions.md.

## 9.4 Out of scope

Not included in this firmware: Heltec gateway and Jetson receiver software, over-the-air updates, permanent device identity, packet encryption/authentication, and battery monitoring (the PCB has no rail or battery measurement input).

# 10. Bring-up Procedure

Run idf.py set-target esp32s3, then idf.py menuconfig ("CowNect collar configuration"), then idf.py build.

In main/app_main.cpp, uncomment one test, then flash and open the serial monitor.

Read the result line (PASS, FAIL, BLOCKED or NEEDS_HARDWARE_VALIDATION) and record it in BUILD_AND_VALIDATION_REPORT.md.

Work through Material 4 (board) to Material 15 (hybrid) in order, then enable run_cownect_firmware().

Alternatively, flash the final firmware with the PROG switch set to PROGRAMMING and run the same tests interactively from the console (for example test board, test capture 30, test lora_spi).